# IMAS (Intelligent Mobility Assistance System)

## Technical Architecture & System Specification Document

### Version 1

# 1. Introduction

## Project Name

IMAS (Intelligent Mobility Assistance System)

## Purpose

IMAS is an AI-powered embedded safety system designed for heavy vehicles such as:

- Trucks

- Buses

- Commercial Fleet Vehicles

- Mining Vehicles

- Industrial Transport Vehicles

The system continuously monitors:

### Driver Behaviour

- Drowsiness

- Fatigue

- Yawning

- Phone Usage

- Driver Distraction

### Road Conditions

- Vehicle Proximity

- Collision Risk

- Approaching Vehicle Risk

### Vehicle Status

- GPS Tracking

- Driver Safety Score

- Historical Risk Trends

The primary objective is to reduce accidents caused by:

- Driver Fatigue

- Driver Distraction

- Human Error

- Delayed Reaction

# 2. System Architecture

## Hardware Components

### Driver Monitoring Camera

Purpose:

Monitor driver behaviour.

Detect:

- Eye Closure

- Yawning

- Phone Usage

Patentable:

No

Reason:

Camera-based driver monitoring is an existing technology.

### Road Monitoring Camera

Purpose:

Monitor road environment.

Detect:

- Vehicles

- Distance

- Relative Movement

Patentable:

No

Reason:

Object detection using vision systems is already established.

### NXP Processor

Purpose:

Execute:

- YOLO

- OpenCV

- Risk Engine

- Alert Logic

Patentable:

No

Reason:

Hardware platform selection is not patentable.

### GPS Module

Purpose:

Live vehicle tracking.

Patentable:

No

Reason:

Standard GPS tracking implementation.

### Speaker/Buzzer

Purpose:

Real-time driver alerts.

Patentable:

No

Reason:

Standard alert mechanism.

# 3. Software Architecture

Main Software Modules:

Driver Camera

↓

Detection Engine

↓

Event Engine

↓

Risk Prediction Engine

↓

Alert Escalation Engine

↓

Driver Score Engine

↓

Dashboard / Mobile App

# 4. Original IMAS Features (Before Improvements)

# 4.1 Eye Closure Detection

## Purpose

Detect driver drowsiness.

## Technology

- OpenCV

- Facial Landmark Detection

- EAR (Eye Aspect Ratio)

## Mathematical Formula

EAR:

EAR = (||p2-p6|| + ||p3-p5||)

-----------------------

2 × ||p1-p4||

Where:

- p1–p6 are eye landmark points.

## Logic

### Calibration Phase

Duration:

6 seconds

System captures:

Normal EAR

and stores baseline.

### Detection Phase

If:

EAR < Baseline Threshold

continuously for:

3 seconds

Then:

Sleepiness Event

generated.

Patentable:

No

Reason:

EAR-based drowsiness detection is already widely published.

# 4.2 Yawning Detection

## Purpose

Detect fatigue.

## Technology

MAR (Mouth Aspect Ratio)

## Formula

MAR =

Vertical Mouth Distance

-----------------------

Horizontal Mouth Distance

## Logic

If:

MAR > Threshold

for:

3 seconds

Generate:

Yawning Event

Patentable:

No

Reason:

MAR-based fatigue detection already exists.

# 4.3 Phone Usage Detection

## Technology

YOLO

Dataset:

COCO Dataset

Classes:

- Mobile Phone

- Person

## Logic

If:

Phone

+

Driver

detected together

Generate:

Phone Usage Event

Patentable:

No

Reason:

Standard object detection.

# 4.4 Road Vehicle Detection

## Technology

YOLO

Detect:

- Car

- Truck

- Bus

- Motorcycle

## Distance Categories

System Classifies:

Far

Medium

Close

Critical

using bounding box measurements.

Patentable:

No

Reason:

Distance classification using object size is established.

# 4.5 GPS Tracking

Purpose:

Live vehicle location monitoring.

Patentable:

No

Reason:

Standard GPS implementation.

# 4.6 Driver Alerts

If any event occurs:

Sleepy

Yawning

Phone Usage

Close Vehicle

Critical Vehicle

Speaker generates:

Beep Alert

Patentable:

No

Reason:

Generic warning mechanism.

# 5. Weaknesses Identified in Original IMAS

- No behaviour prediction.

- No risk accumulation.

- No event severity scoring.

- No alert prioritization.

- No trend analysis.

- No driver scoring.

- No adaptive baseline updates.

- No advanced logging.

# 6. Improvements Added to IMAS V1

# 6.1 Risk Prediction Engine

## Purpose

The original IMAS generated alerts only when individual events occurred.

Example:

Eye Closure → Alert

Yawning → Alert

Phone Usage → Alert

The system could not predict future risk.

The Risk Prediction Engine was introduced to identify dangerous behaviour patterns before an accident occurs.

## Concept

Instead of evaluating individual events, IMAS evaluates accumulated risk over time.

Two independent risk categories are maintained:

### Fatigue Risk

Related to:

- Eye Closure

- Yawning

### Distraction Risk

Related to:

- Phone Usage

- Close Vehicle

- Critical Vehicle

## Fatigue Event Scoring

| Event | Score |
| --- | --- |
| Yawning | +5 |
| Sleepiness (Eye Closure) | +20 |
| Consecutive Eye Closure | +30 |

## Distraction Event Scoring

| Event | Score |
| --- | --- |
| Phone Usage | +25 |
| Close Vehicle | +15 |
| Critical Vehicle | +40 |
| Repeated FCW Event | +20 |

## Rolling Risk Window

The system maintains a:

2 Minute Rolling Window

All event scores occurring inside the window are accumulated.

## Risk Levels

### Level 1

0 – 30

Status:

Normal

### Level 2

31 – 60

Status:

Trend Detected

### Level 3

61 – 99

Status:

High Risk

### Level 4

100+

Status:

Critical Risk

## Example

Yawning = 5

Yawning = 5

Sleepiness = 20

Fatigue Score:

30

No alert.

Later:

Sleepiness = 20

New Score:

50

Trend Alert Generated.

Patentability:  Potentially Patentable

Reason: Risk accumulation logic combined with fatigue and distraction fusion can be drafted as a system-level invention.

# 6.2 Adaptive Baseline Update

## Problem

Original system calibration:

6 Seconds

only during startup.

Long driving sessions may change:

- Driver posture

- Camera angle

- Lighting

- Face position

leading to reduced accuracy.

## Solution

Adaptive Baseline Update.

The baseline is updated only during safe-driving periods.

## Conditions

For:

5 Minutes

all conditions must be satisfied:

- No Eye Closure Alerts

- No Yawning Alerts

- No Phone Usage Alerts

- Stable Face Detection

## Formula

EAR:

EAR_new =

0.95 × EAR_old

+

0.05 × EAR_current

MAR:

MAR_new =

0.95 × MAR_old

+

0.05 × MAR_current

## Advantage

Prevents:

Sleepy State

→ Becoming New Baseline

while allowing gradual adaptation.

Patentability: Potentially Patentable

Reason: Condition-based adaptive calibration for driver-specific fatigue monitoring has patent potential.

# 6.3 Closing Speed Detection

## Problem

Distance alone does not indicate risk.

Example:

20m Distance

may be safe or dangerous depending on approach rate.

## Solution

Use existing vehicle speed and bounding-box growth information.

The system evaluates:

Distance

+

Approach Speed

simultaneously.

## Logic

Vehicle approaching rapidly:

Bounding Box Growth Rate ↑

Risk increases.

## Benefit

Differentiates:

Close but Stable

from

Close and Rapidly Approaching

Patentability: Weak

Reason: Closing-speed estimation itself is well known.

# 6.4 Vehicle Event Severity Scoring

## Purpose

Convert road events into measurable risk values.

## Scoring

| Event | Score |
| --- | --- |
| Far | 0 |
| Medium | 5 |
| Close | 15 |
| Critical | 40 |

## Additional Closing Speed Contribution

| Speed Level | Score |
| --- | --- |
| Normal | 0 |
| Moderate | 10 |
| High | 20 |

## Example

Vehicle:

Close = 15

High Closing Speed = 20

Total:

35

added to distraction risk.

Patentability: Potentially Patentable

Reason: Risk scoring fusion of distance and approach speed may contribute to larger system claims.

# 6.5 Alert Escalation Logic

## Purpose

Avoid notification spam.

Notify owners only for meaningful situations.

## Level 1

Event Detected:

- Yawning

- Sleepiness

- Phone Usage

- Close Vehicle

- Critical Vehicle

Action:

Driver Beep Only

## Level 2

Repeated events.

Action:

Driver Beep Only

Risk Score Increased.

## Level 3

Threshold reached.

Owner receives:

### Fatigue Alert

Driver is Getting Sleepy

Take Immediate Action

Sub Events: Yawning, Sleepiness

### Distraction Alert

Driver is Getting Distracted Take Immediate Action

Sub Events: Phone Usage, Close Vehicle, Critical Vehicle

## Level 4

Risk remains above threshold for:

5 Minutes

Generate:

Critical Fatigue Alert

or

Critical Distraction Alert

## Cooldown Logic

Repeated Behaviour Alerts: 10 Minute Cooldown implemented.

Prevents owner notification spam.

Patentability: Strong Potential

Reason: Multi-level risk escalation based on persistence and behavioural trends is significantly more novel than simple alerts.

# 6.6 History-Based Trend Analysis

## Purpose

Identify repeated dangerous behaviour.

## Rolling History

30 Minutes maintained.

## Tracked Parameters

- Fatigue Alerts

- Distraction Alerts

- Phone Usage Events

- Eye Closure Events

- Yawning Events

- Close Vehicle Events

- Critical Vehicle Events

## Trend Logic

If:

3+

Fatigue Alerts

within 30 min

Generate:

Repeated Fatigue Behaviour

If: 3+ Distraction Alerts within 30 min

Generate: Repeated Distraction Behaviour

Patentability: Strong Potential

Reason: Behaviour trend modelling combined with driver monitoring can strengthen patent claims.

# 6.7 Driver Safety Score

## Purpose

Provide a simple fleet-management metric.

## Score Range

0 – 100

## Initial Score

100

## Deductions

| Event | Deduction |
| --- | --- |
| Level 3 Fatigue Alert | -5 |
| Level 4 Fatigue Alert | -10 |
| Level 3 Distraction Alert | -5 |
| Level 4 Distraction Alert | -10 |
| Repeated Phone Behaviour | -5 |
| Repeated Critical Vehicle Behaviour | -5 |
| Camera Blocked | -10 |
| Camera Tampering | -20 |

## Recovery Logic

For every:

30 Minutes

of safe driving:

+2 Points

Maximum:

100

## Driver Ratings

| Score | Rating |
| --- | --- |
| 90-100 | Excellent |
| 75-89 | Good |
| 60-74 | Average |
| 40-59 | Needs Attention |
| 0-39 | High Risk |

## Analytics

Stored:

- Daily Score

- Weekly Score

- Monthly Score

Patentability: Potentially Patentable

Reason: Safety scoring integrated with AI-based fatigue/distraction analytics may be patentable as part of the overall system.

# 6.8 Enhanced Alert Logging

## Purpose

Provide traceability and analytics.

## Stored Information

For every major alert:

- Timestamp

- Alert Type

- Trigger Events

- Risk Score

- Risk Level

- Driver Score

- GPS Location

## Example

Timestamp:

10:45 AM

Alert:

Fatigue Alert

Risk Score:

72

Risk Level:

High

Driver Score:

84

GPS:

12.9716,77.5946

Patentability:

Generally Not Patentable

Reason:

Logging itself is standard practice.

However, it strengthens evidence and supports patentable system features.

# 7. Patentability Summary

| Feature | Patentability |
| --- | --- |
| EAR Detection | ❌ |
| MAR Detection | ❌ |
| YOLO Detection | ❌ |
| GPS Tracking | ❌ |
| Vehicle Distance Detection | ❌ |
| Risk Prediction Engine | ✅ |
| Adaptive Baseline Update | ✅ |
| Vehicle Severity Scoring | ✅ |
| Alert Escalation Logic | ✅ |
| Trend Analysis | ✅ |
| Driver Safety Score | ✅ |
| Enhanced Logging | ❌ |
| Combined Risk Fusion Architecture | ✅✅ Strongest |